# Summary: Discrete Fourier Transform and FFT Analysis

## Course: Signals and Systems

Department of Electrical Engineering

National University of Computer and Emerging Sciences, Lahore

| Submitted by: | Zainab Bilal (Roll Number: 24l-6127) |
| --- | --- |
| Submitted to: | Sugrah Kamran |
| Date of Submission: | 29/4/2026 |

## 1. Introduction

The Discrete Fourier Transform (DFT) is used to convert a discrete-time signal into its frequency-domain representation. It helps identify frequency components present in the signal. In this experiment, a random signal of length N=16 is generated based on roll number digits, and both manual DFT and MATLAB FFT are implemented. This experiment focuses on manually implementing the DFT and comparing it with MATLAB’s built-in FFT function to explore frequency resolution, spectral leakage, zero-padding, and windowing techniques.

## 2. Problem Analysis

- Main Objectives: Compute DFT manually and compare with FFT, analyse frequency resolution, study spectral leakage, verify symmetry for real signals, and apply windowing (Hamming) to reduce leakage.

- The Challenge: Implement DFT without loops using the matrix method and interpret differences between theoretical and computed results.

- Key Issues: DFT assumes periodic signals which causes leakage. Finite length signals distort frequency representation, making it necessary to verify theoretical concepts using simulation.

## 3. Design Requirements

Signal Definition: Given roll-number-based length: N = 6 + 1 + 2 + 7 = 16

Discrete-time signal: x[n] = rand(1,16), for n = 0, 1, 2, ..., 15

Analysis Framework: Compare X_fft and X_dft, and thoroughly analyze Zero-padding (Nz), Leakage (integer vs non-integer frequency), and Windowing (Hamming).

## 4. Feasibility Analysis

The system is fully feasible because MATLAB efficiently handles matrix operations, and the small signal size (N=16) minimizes computation time and memory requirements. FFT provides a perfect built-in validation mechanism, and visualization tools (subplot, stem) assist in comprehensive analysis. Limitations: Manual DFT becomes inefficient for large N, and leakage cannot be completely removed.

## 5. Possible Solutions

- Approach 1: Direct DFT using mathematical definition loop execution with O(N²) complexity. Accurate but slow.

- Approach 2: FFT (Fast Fourier Transform) using O(N log N) computational steps. Fast and practical.

- Approach 3: Windowing + Zero Padding to improve spectral quality. Used extensively in real-world systems.

- Final Solution: A combination of all methods as provided in the integrated simulation code.

## 6. Preliminary Design & Flowchart

Start -> Compute N (sum of digits) -> Generate x[n] -> Compute FFT -> 
Construct DFT Matrix -> Compare Results -> Apply Zero Padding -> 
Generate Signals (f=2, 2.5) -> Analyze Leakage -> Apply Hamming Window -> Plot Results -> End

## 7. Design Description & Mathematical Modeling

DFT Matrix Construction: WN(k,n) = W^(k*n), where W = e^(-j*2*pi/16) = cos(2*pi/16) - j*sin(2*pi/16)

Manual Calculation (Example for k = 2): X[2] = x[0] + x[1]W² + x[2]W⁴ + ... + x[15]W³⁰

Zero Padding: Nz = 64. Resolution Δf before padding = 1/16 = 0.0625; Resolution after padding = 1/64 = 0.0156.

Spectral Leakage: Case 1 (No Leakage) at f=2 shows perfect alignment and a sharp peak. Case 2 (Leakage) at f=2.5 creates boundary misalignment where energy spreads across bins.

Windowing: Applied via w[n] = 0.54 - 0.46*cos(2*pi*n/15) to reduce sharp discontinuities and smooth edges.

## 8. Software Simulation

clc;
clear;
N = 16; % 6+1+2+7
n = 0:N-1;
x = rand(1, N);
k = n';
W = exp(-j * 2 * pi / N);
WN = W .^ (k * n); % DFT matrix
X_DFT = WN * x'; % Matrix multiplication
X_fft = fft(x);

Nz = 64;
X_zp = fft(x, Nz);
f1 = 2; % integer frequency
f2 = 2.5; % non-integer frequency
x1 = sin(2*pi*f1*n/N);
x2 = sin(2*pi*f2*n/N);
X1 = fft(x1);
X2 = fft(x2);

## 9. Experimental Plots Layout Summary

The simulation generates 6 detailed subplots within a single interface window to visualize the performance metrics:

- Subplot 1: Magnitude Spectrum: Baseline discrete frequency distribution.

- Subplot 2: Zero-Padded Spectrum: Smooth interpolation profile across 64 bins.

- Subplot 3: No Leakage: Concentrated sharp peak at index position k=2.

- Subplot 4: Spectral Leakage: Distributed spectrum showing side-lobe bleeding at f=2.5.

- Subplot 5: Hamming Window: Attenuated sidelobes with smoother peak edges.

- Subplot 6: Manual DFT: Absolute identity match proving manual matrix computation matches FFT.

## 10. Performance Analysis

| Method | Complexity | Speed | Accuracy |
| --- | --- | --- | --- |
| Manual DFT | O(N²) | Slow | High |
| FFT | O(N log N) | Fast | High |

### Key Insights

- FFT vs DFT: FFT is a fast version of DFT yielding identical results.

- Zero Padding: Improves visual resolution but does NOT increase actual data information.

- Leakage & Windowing: Leakage depends on signal frequency and is caused by boundary discontinuity. Windowing reduces sidelobes at the cost of wider peaks.

## 11. Future Scope, Social and Cultural Implications

Advanced discrete frequency tracking formats the core engine behind modern 5G cellular communication channels, audio compression layout structures (MP3), and biomedical scanning electronics (EEG/ECG signal conditioning).

## 12. Conclusion

The experiment successfully demonstrates the manual implementation of the Discrete Fourier Transform and validates it against MATLAB’s FFT. The results confirm theoretical expectations through simulation and comparative analysis.

## 13. References

- Signals and Systems – Alan V. Oppenheim, Alan S. Willsky, Prentice Hall.

- MATLAB Documentation – FFT and Window Functions.